PROBLEM SYMPTOMS TABLE

Symptom Suspect Area See page

Engine does not crank (Does not start)

6. Starter

7. Starter relay

ST-15

ST-17

No initial combustion (Does not start)

1. Engine immobiliser system

2. ECM power source circuit

3. Fuel pump control circuit

DI-884

DI-331

DI-335

No complete combustion (Does not start) 1. Fuel pump control circuit DI-335

Under normal condition (Difficult to start)

1. Starter signal circuit

2. Fuel pump control circuit

3. Compression

DI-328

DI-335

EM-3

Cold engine (Difficult to start)

1. Starter signal circuit

2. Fuel pump control circuit

DI-328

DI-335

Hot engine (Difficult to start)

1. Starter signal circuit

2. Fuel pump control circuit

DI-328

DI-335

High engine idle speed (Poor idling)

1. A/C signal circuit (Compressor circuit)

2. ECM power source circuit

DI-1300

DI-331

Low engine idle speed (Poor idling)

1. A/C signal circuit (Compressor circuit)

2. Fuel pump control circuit

DI-1300

DI-335

Rough idling (Poor idling)

1. Compression

2. Fuel pump control circuit

EM-3

DI-335

Hunting (Poor idling)

1. ECM power source circuit

2. Fuel pump control circuit

DI-331

DI-335

Hesitation/Poor acceleration (Poor driveability)

1. Fuel pump control circuit

2. A/T faulty

DI-335

DI-435

Surging (Poor driveability) 1. Fuel pump control circuit DI-335

Soon after starting (Engine stall)

1. Engine immobiliser system

2. Fuel pump control circuit

DI-884

DI-335

During A/C operation (Engine stall)

1. A/C signal circuit (Compressor circuit)

2. ECM

DI-1300

IN-22

Unable to refuel/Difficult to refuel 1. ORVR system