| Last Modified: 2-23-2009 | 5.7 U | From: 200607 |
| --- | --- | --- |
| Model Year: 2007 | Model: GS350 | Doc ID: RM0000011BQ00YX |
| Title: AUDIO / VISUAL: AUDIO AND VISUAL SYSTEM: TERMINALS OF ECU (2007 GS350 GS430) |  |  |

## TERMINALS OF ECU

1. CHECK MULTI-DISPLAY

(a) Disconnect the K46 display connector.

(b) Measure the resistance and voltage of the wire harness side connector.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specified Condition |
| --- | --- | --- | --- | --- |
| +B1 (K46-1) - GND1 (K46-10) | O - W-B | Battery | Always | 10 to 14 V |
| ACC (K46-2) - GND1 (K46-10) | GR - W-B | Accessory (ON) | Engine switch off | Below 1 V |
| ACC (K46-2) - GND1 (K46-10) | GR - W-B | Accessory (ON) | Engine switch on (ACC) | 10 to 14 V |
| IG (K46-3) - GND1 (K46-10) | B - W-B | Ignition (ON) | Engine switch off | Below 1 V |
| IG (K46-3) - GND1 (K46-10) | B - W-B | Ignition (ON) | Engine switch on (IG) | 10 to 14 V |
| GND1 (K46-10) - Body ground | W-B - Body ground | Ground | Always | Below 1 Ω |

  If the result is not as specified, there may be a malfunction on the wire harness side.

(c) Reconnect the K46 display connector.

(d) Measure the voltage of the connector.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specified Condition |
| --- | --- | --- | --- | --- |
| TC (K46-7) - GND1 (K46-10) | BR - W-B | Diagnosis ON signal | Engine switch on (IG) | 9 to 14 V |
| ILL- (K46-11) - GND1 (K46-10) | L - W-B | Illumination signal | Light control switch OFF | Below 1 V |
| ILL- (K46-11) - GND1 (K46-10) | L - W-B | Illumination signal | Light control switch ON | 10 to 14 V |
| ILL+ (K46-12) - GND1 (K46-10) | G - W-B | Illumination signal | Light control switch OFF | Below 1 V |
| ILL+ (K46-12) - GND1 (K46-10) | G - W-B | Illumination signal | Light control switch ON | 10 to 14 V |
| REV (K46-13) - GND1 (K46-10) | R - W-B | Reverse signal | See "Vehicle Signal Check" Mode | - |
| SPD (K46-14) - GND1 (K46-10) | V - W-B | Speed signal from combination meter | See "Vehicle Signal Check" Mode | - |
| PKB (K46-23) - GND1 (K46-10) | Y - W-B | Parking brake signal | See "Vehicle Signal Check" Mode | - |
| TSW- (K47-5) - GND1 (K46-10) | W-B - W-B | Telephone relation switch signal | Always | Below 1 V |
| MACC (K47-9) - GND1 (K46-10)*2 | O - W-B | Microphone amplifier power supply | Engine switch off | Below 1 V |
| MACC (K47-9) - GND1 (K46-10)*2 | O - W-B | Microphone amplifier power supply | Engine switch on (IG) | 5 V |
| MIN- (K47-10) - GND1 (K46-10) | R - W-B | Microphone voice signal | See "Microphone & Voice Recognition Check" | - |
| MIN+ (K47-11) - GND1 (K46-10) | G - W-B | Microphone voice signal | See "Microphone & Voice Recognition Check" | - |
| CGND (K47-14) - Body ground | W - Body ground | Ground | Always | Below 1 V |
| IVO- (K47-21) - GND1 (K46-10) | BR - W-B | Voice guidance signal | Voice guidance is provided | Waveform synchronized with sounds is output |
| IVO+ (K47-22) - GND1 (K47-10) | Y - W-B | Voice guidance signal | Voice guidance is provided | Waveform synchronized with sounds is output |
| SLD (K47-23) - GND1 (K46-10) | Shielded - W-B | Shielded ground | Always | Below 1 V |
| VV+ (x2-1) - GND1 (K46-10)*1 | W - W-B | DVD image signal | DVD is played on radio receiver | Pulse generation (refer to waveform 2) |
| VV- (x2-4) - GND1 (K46-10)*1 | B - W-B | Signal ground | Always | Below 1 V |
| CA+ (K47-13) - GND1 (K46-10) | B - W-B | Television camera power supply | Engine switch on (IG), shift lever reverse position | Approx. 6 V |
| CGND (K46-14) - Body ground | W - Body ground | Ground | Always | Below 1 V |
| V+ (K47-27) - CGND (K47-14) | R - W | Television camera image signal | Engine switch on (IG), shift lever reverse position | Pulse generation (refer to waveform 1) |
| V- (K47-28) - CGND (K47-14) | Shielded - W | Shielded ground | Always | Below 1 V |

HINT:

*1: for DVD Changer Models

*2: w/o LEXUS Link System

  If the result is not as specified, the multi-display may have a malfunction.

2. CHECK RADIO RECEIVER

(a) Disconnect the K54 receiver connector.

(b) Measure the resistance and voltage of the wire harness side connector.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specification |
| --- | --- | --- | --- | --- |
| B (K54-1) - GND (K54-20) | SB - BR | Battery | Always | 10 to 14 V |
| GND (K54-20) - Body ground | BR - Body ground | Ground | Always | Below 1 Ω |
| ACC (K54-11) - GND (K54-20) | GR - BR | Accessory (ON) | Engine switch off | Below 1 V |
| ACC (K54-11) - GND (K54-20) | GR - BR | Accessory (ON) | Engine switch on (ACC) | 10 to 14 V |

  If the result is not as specified, there may be a malfunction on the wire harness side.

(c) Reconnect the K54 receiver connector.

(d) Measure the voltage on the connectors.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specification |
| --- | --- | --- | --- | --- |
| ILL+ (K54-2) - GND (K54-20) | G - BR | Illumination signal | Engine switch on (IG), light control switch OFF | Below 1 V |
| ILL+ (K54-2) - GND (K54-20) | G - BR | Illumination signal | Engine switch on (IG), light control switch TAIL or ON | 10 to 14 V |
| MUTE (K54-7) - GND (K54-20) | SB - BR | Mute signal | Audio system is playing → changing mode | Above 3.5 V → Below 1 V |
| R+ (K54-8) - GND (K54-20) | R - BR | Sound signal (right) | Audio system is playing | Waveform synchronized with sound is output |
| L+ (K54-9) - GND (K54-20) | B - BR | Sound signal (left) | Audio system is playing | Waveform synchronized with sound is output |
| R- (K54-18) - GND (K54-20) | G - BR | Sound signal (right) | Audio system is playing | Waveform synchronized with sound is output |
| L- (K54-19) - GND (K54-20) | W - BR | Sound signal (left) | Audio system is playing | Waveform synchronized with sound is output |
| ILL- (K54-12) - GND (K54-20) | W - BR | Illumination signal | Engine switch off | Below 1 V |
| ILL- (K54-12) - GND (K54-20) | W - BR | Illumination signal | Engine switch on (IG) | Pulse generation |
| TX+ (K56-9) - GND (K54-20) | L - BR | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| TX- (K56-10) - GND (K54-20) | LG - BR | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| TXM+ (K53-9) - GND (K54-20) | L - BR | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| TXM- (K53-10) - GND (K54-20) | LG - BR | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| ATX+ (K54-5) - GND (K54-20) | L - BR | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| ATX- (K54-15) - GND (K54-20) | LG - BR | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| ANT (K54-13) - GND (K54-20) | O - BR | Power source of antenna | Radio switch is ON and AM or FM | 10 to 14 V |
| SWG (K56-6) - GND (K54-20) | LG - BR | Steering pad switch ground | Always | Below 1 V |
| SW1 (K56-7) - GND (K54-20) | Y - BR | Steering pad switch signal | Steering pad switch not pushed | 4 V or more |
| SW1 (K56-7) - GND (K54-20) | Y - BR | Steering pad switch signal | Steering pad switch SEEK+ switch pushed | Below 2.5 V |
| SW1 (K56-7) - GND (K54-20) | Y - BR | Steering pad switch signal | Steering pad switch SEEK- switch pushed | Approx. 0.9 V |
| SW1 (K56-7) - GND (K54-20) | Y - BR | Steering pad switch signal | Steering pad switch Vol+ switch pushed | Approx. 1.5 V |
| SW1 (K56-7) - GND (K54-20) | Y - BR | Steering pad switch signal | Steering pad switch Vol+ switch pushed | Approx. 0.9 V |
| SW2 (K56-8) - GND (K54-20) | O - BR | Steering pad switch signal | Steering pad switch not pushed | 4 V or more |
| SW2 (K56-8) - GND (K54-20) | O - BR | Steering pad switch signal | Steering pad switch MODE switch pushed | Below 2.5 V |
| SW2 (K56-8) - GND (K54-20) | O - BR | Steering pad switch signal | Steering pad switch DISP switch pushed | Below 2.5 V |
| NTSC (x1-1) - GND (K54-20)* | W - BR | DVD image signal | DVD played | Pulse generation (see waveform1) |
| GND (x1-2) - GND (K54-20)* | B - BR | Ground | Always | Below 1 V |
| SLD (x1-3) - GND (K54-20) | Shielded - BR | Shielded ground | Always | Below 1 V |

HINT:

*: for DVD changer models

  If the result is not as specified, the receiver may have a malfunction.

|  | (e) Using an oscilloscope, check the waveform. ## Waveform 1 (reference) Item; Content. Symbols (Terminal No.); NTSC (x1-1) - Body ground. Tool; 200 mV/DIV., 10 μs/DIV.. Condition; DVD displayed on multi-display. |
| --- | --- |

3. CHECK STEREO COMPONENT AMPLIFIER (for CD Changer Models)

(a) Disconnect the U29 and U42 amplifier connectors.

(b) Measure the resistance and voltage of the connectors.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specified Condition |
| --- | --- | --- | --- | --- |
| GND (U42-2) - Body ground | W-B - Body ground | Ground | Always | Below 1 Ω |
| GND2 (U42-6) - Body ground | W-B - Body ground | Ground | Always | Below 1 Ω |
| +B (U42-1) - GND (U42-2) | SB - W-B | Battery | Always | 10 to 14 V |
| +B2 (U42-3) - GND (U42-2) | SB - W-B | Battery | Always | 10 to 14 V |
| ACC (U29-12) - GND (U42-2) | GR - W-B | Accessory (ON) | Engine switch off | Below 1 V |
| ACC (U29-12) - GND (U42-2) | GR - W-B | Accessory (ON) | Engine switch on (ACC) | 10 to 14 V |

  If the result is not as specified, there may be a malfunction on the wire harness.

(c) Reconnect the U29 and U42 amplifier connectors.

(d) Measure the voltage of the connectors.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specified Condition |
| --- | --- | --- | --- | --- |
| CTR+ (U44-3) - GND (U42-2) | Y - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| CTR- (U44-9) - GND (U42-2) | B - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| FL+ (U30-2) - GND (U42-2) | P - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| FL- (U30-6) - GND (U42-2) | V - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| FR+ (U30-8) - GND (U42-2) | LG - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| FR- (U30-7) - GND (U42-2) | L - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| RL+ (U30-3) - GND (U42-2) | B - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| RL- (U30-9) - GND (U42-2) | Y - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| RR+ (U30-4) - GND (U42-2) | R - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| RR- (U30-10) - GND (U42-2) | W - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFL+ (U44-1) - GND (U42-2) | W - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFL- (U44-5) - GND (U42-2) | R - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFR+ (U44-2) - GND (U42-2) | G - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFR- (U44-6) - GND (U42-2) | W - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| II1+ (U29-15) - GND (U42-2) | Y - W-B | Voice guidance signal | Voice guidance is proved | Waveform synchronized with sound is output |
| II1- (U29-14) - GND (U42-2) | BR - W-B | Voice guidance signal | Voice guidance is proved | Waveform synchronized with sound is output |
| SPD (U29-11) - GND (U42-2) | V - W-B | Speed signal | Engine switch on (IG), drive wheels are turned slowly | Pulse generation |
| MUTE (U29-1) - GND (U42-2) | O - W-B | Mute signal | Audio system is playing → change mode | Above 3.5 V → Below 1 V |
| TX+ (U29-8) - GND (U42-2) | LG - W-B | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| TX- (U29-7) - GND (U42-2) | L - W-B | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| R+ (U29-5) - GND (U42-2) | R - W-B | Sound signal (right) | Audio system playing | Waveform synchronized with sound is output |
| R- (U29-4) - GND (U42-2) | G - W-B | Sound signal (right) | Audio system playing | Waveform synchronized with sound is output |
| L+ (U29-3) - GND (U42-2) | B - W-B | Sound signal (left) | Audio system playing | Waveform synchronized with sound is output |
| L- (U29-2) - GND (U42-2) | W - W-B | Sound signal (left) | Audio system playing | Waveform synchronized with sound is output |
| SLD (U29-6) - GND (U42-2) | Shielded - Body ground | Shielded ground | Always | Below 1 V |

  If the result is not as specified, the amplifier may have a malfunction.

4. CHECK STEREO COMPONENT AMPLIFIER (for DVD Changer Models)

(a) Disconnect the U31 amplifier connector.

(b) Measure the resistance and voltage of the connector.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specified Condition |
| --- | --- | --- | --- | --- |
| GND (U31-16) - Body ground | W-B - Body ground | Ground | Always | Below 1 Ω |
| GND2 (U31-15) - Body ground | W-B - Body ground | Ground | Always | Below 1 Ω |
| +B (U31-1) - GND (U31-16) | SB - W-B | Battery | Always | 10 to 14 V |
| +B2 (U31-10) - GND (U31-16) | SB - W-B | Battery | Always | 10 to 14 V |
| ACC (U29-12) - GND (U31-16) | GR - W-B | Accessory (ON) | Engine switch off | Below 1 V |
| ACC (U29-12) - GND (U31-16) | GR - W-B | Accessory (ON) | Engine switch on (ACC) | 10 to 14 V |

  If the result is not as specified, there may be a malfunction on the wire harness.

(c) Reconnect the U31 amplifier connector.

(d) Measure the voltage of the connectors.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specified Condition |
| --- | --- | --- | --- | --- |
| MUTE (U29-1) - GND (U31-16) | O - W-B | Mute signal | Audio system is sounding → Changing mode | Above 3.5 V → Below 1 V |
| L- (U29-2) - GND (U31-16) | W - W-B | Sound signal (left) | Audio system is playing | Waveform synchronized with sounds is output |
| L+ (U29-3) - GND (U31-16) | B - W-B | Sound signal (left) | Audio system is playing | Waveform synchronized with sounds is output |
| R- (U29-4) - GND (U31-16) | G - W-B | Sound signal (right) | Audio system is playing | Waveform synchronized with sounds is output |
| R+ (U29-5) - GND (U31-16) | R - W-B | Sound signal (right) | Audio system is playing | Waveform synchronized with sounds is output |
| TX- (U29-7) - GND (U31-16) | L - W-B | AVC-LAN communication signal | Engine switch is on (ACC) | 2 to 3 V |
| TX+ (U29-8) - GND (U31-16) | LG - W-B | AVC-LAN communication signal | Engine switch is on (ACC) | 2 to 3 V |
| SPD (U29-11) - GND (U31-16) | V - W-B | Speed signal | Engine switch is on (IG), drive wheels are turned slowly | Pulse generation |
| FL+ (U31-8) - GND (U31-16) | P - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| FR- (U31-20) - GND (U31-16) | L - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| RL- (U31-17) - GND (U31-16) | Y - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| RR- (U31-18) - GND (U31-16) | W - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| FL- (U31-19) - GND (U31-16) | V - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| FR+ (U31-9) - GND (U31-16) | LG - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| RL+ (U31-6) - GND (U31-16) | B - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| RR+ (U31-7) - GND (U31-16) | R - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| WF1+ (U31-5) - GND (U31-16) | P - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| WF1- (U31-14) - GND (U31-16) | V - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| WF2+ (U31-4) - GND (U31-16) | L - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| WF2- (U31-13) - GND (U31-16) | LG - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| CTR+ (U31-3) - GND (U31-16) | Y - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| CTR- (U31-12) - GND (U31-16) | B - W-B | Sound signal | Audio system is playing | Waveform synchronized with sounds is output |
| SL+ (U30-1) - GND (U31-16) | LG - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| SL- (U30-5) - GND (U31-16) | L - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| SR+ (U30-2) - GND (U31-16) | B - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| SR- (U30-6) - GND (U31-16) | Y - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFL+ (U30-3) - GND (U31-16) | W - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFL- (U30-9) - GND (U31-16) | R - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFR+ (U30-4) - GND (U31-16) | G - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |
| WFR- (U30-10) - GND (U31-16) | W - W-B | Sound signal | Audio system is playing | Waveform synchronized with sound is output |

  If the result is not as specified, the amplifier may have a malfunction.

5. CHECK NETWORK GATEWAY ECU

(a) Measure the voltage of the connector.

| Symbols (Terminal No.) | Wiring Color | Terminal Description | Condition | Specified Condition |
| --- | --- | --- | --- | --- |
| GTX+ (K66-6) - Body ground | L - Body ground | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |
| GTX- (K66-21) - Body ground | LG - Body ground | AVC-LAN communication signal | Engine switch on (ACC) | 2 to 3 V |