Keyless Entry System

User’s Manual

Parameter:

1. Voltage: +12V±2V(DC)

2. Static current: ≤8mA

3. Max current: 15A

4. Direction output≤15A, central lock output≤15A, trunk output≤-500mA

5. Frequency:315MHz

Function & Operation

- Lock

Press  shortly, the direction lights will flash one time and the car will be locked.

- Unlock

Press shortly, the direction lights will flash two times and the car will be unlocked.

- Car location

Press  for 1 second when the car is locked, the direction lights will flash 25 seconds to help you search the car. Press  or shortly to stop it.

- Trunk release

Press   for 2 seconds when the car is unlocked, the trunk will be released.

- Electric lock & air lock

Electric lock needs 0.5 seconds to work and air lock needs 3.5 seconds. You can choose the right one for your car.

- Learn code

When keep pressing the white button on mainframe board, press any button on remote to learn new code for remote and the direction lights will flash at the same time.

Diagram: