EM–44

ENGINE MECHANICAL

–

CYLINDER HEAD

EM09S–02

INSPECTION

1.

(a)

CLEAN TOP SURFACES OF PISTONS AND CYL-

INDER BLOCK

Turn the crankshaft, and bring each piston to top dead

center (TDC). Using a gasket scraper, remove all the car-

bon from the piston top surface.

A03194

(b)

Using a gasket scraper, remove all the gasket material

from the cylinder block surface.

(c) Using compressed air, blow carbon and oil from the bolt

holes.

CAUTION:

Protect your eyes when using high pressure compressed

air.

A03193

2.REMOVE GASKET MATERIAL

Using a gasket scraper, remove all the gasket material from the

cylinder block contact surface.

NOTICE:

Be careful not to scratch the cylinder block contact sur-

face.

EM6323

3.CLEAN COMBUSTION CHAMBERS

Using a wire brush, remove all the carbon from the combustion

chambers.

NOTICE:

Be careful not to scratch the cylinder block contact sur-

face.

EM6324

4.CLEAN VALVE GUIDE BUSHINGS

Using a valve guide bushing brush and solvent, clean all the

guide bushings.

P21861

2000 LEXUS LS400 (RM717U)

Author :

Date :

1226

EM–45

ENGINE MECHANICAL

–

CYLINDER HEAD

5.CLEAN CYLINDER HEAD

Using a soft brush and solvent, thoroughly clean the cylinder

head.

EM6325

6.INSPECT FOR FLATNESS

Using a precision straight edge and feeler gauge, measure the

surfaces contacting the cylinder block and the manifolds for

warpage.

Maximum warpage: 0.10 mm (0.0039 in.)

If warpage is greater than maximum, replace the cylinder head.

A02876

A02877

A02878

A02879

7.INSPECT FOR CRACKS

Using a dye penetrant, check the combustion chamber, intake

ports, exhaust ports and cylinder block surface for cracks.

If cracked, replace the cylinder head.

EM6327

8.

(a)

(b)

CLEAN VALVES

Using a gasket scraper, chip off any carbon from the valve

head.

Using a wire brush, thoroughly clean the valve.

EM0580

2000 LEXUS LS400 (RM717U)

Author :

Date :

1227

EM–46

ENGINE MECHANICAL

–

CYLINDER HEAD

9.

(a)

INSPECT VALVE STEMS AND GUIDE BUSHINGS

Using a caliper gauge, measure the inside diameter of the

guide bushing.

Bushing inside diameter:

5.510 – 5.530 mm (0.2169 – 0.2177 in.)

P21862

(b)

Using a micrometer, measure the diameter of the valve

stem.

Valve stem diameter:

Intake

Exhaust

5.470 – 5.485 mm (0.2154 – 0.2159 in.)

5.465 – 5.480 mm (0.2152 – 0.2157 in.)

(c)

Subtract the valve stem diameter measurement from the

guide bushing inside diameter measurement.

Standard oil clearance:

Intake

Exhaust

0.025 – 0.060 mm (0.0010 – 0.0024 in.)

0.030 – 0.065 mm (0.0012 – 0.0026 in.)

Z00052

Maximum oil clearance:

Intake

Exhaust

0.08 mm (0.0031 in.)

0.10 mm (0.0039 in.)

If the clearance is greater than maximum, replace the valve and

guide bushing. (See Page EM–54)

10.

(a)

(b)

INSPECT AND GRIND VALVES

Grind the valve enough to remove pits and carbon.

Check that the valve is ground to the correct valve face

angle.

Valve face angle: 44.5°

44.5°

Z00054

(c)

Check the valve head margin thickness.

Margin thickness:

Standard

Minimum

1.0 mm (0.039 in.)

0.5 mm (0.020 in.)

If the margin thickness is less than minimum, replace the valve.

Margin Thickness

EM0181

2000 LEXUS LS400 (RM717U)

Author :

Date :

1228

EM–47

ENGINE MECHANICAL

–

CYLINDER HEAD

(d)

Check the valve overall length.

Standard overall length:

Intake

95.05 mm (3.7421 in.)

95.10 mm (3.7441 in.)

Exhaust

Overall Length

Minimum overall length:

Intake

Exhaust

94.55 mm (3.7224 in.)

94.60 mm (3.7244 in.)

EM2534

If the overall length is less than minimum, replace the valve.

(e) Check the surface of the valve stem tip for wear.

If the valve stem tip is worn, resurface the tip with a grinder or

replace the valve.

NOTICE:

Do not grind off more than minimum.

EM0255

45° Carbide Cutter

11.

(a)

INSPECT AND CLEAN VALVE SEATS

Using a 45° carbide cutter, resurface the valve seats. Re-

move only enough metal to clean the seats.

EM6330

(b)

Width

Z00055

45°

30°

Check the valve seating position.

Apply a light coat of prussian blue (or white lead) to the

valve face. Lightly press the valve against the seat. Do not

rotate valve.

(c) Check the valve face and seat for the following:

SIf blue appears 360° around the face, the valve is

concentric. If not, replace the valve.

SIf blue appears 360° around the valve seat, the

guide and face are concentric. If not, resurface the

seat.

SCheck that the seat contact is in the middle of the

valve face with the following width:

1.0 – 1.4 mm (0.039 – 0.055 in.)

If not, correct the valve seats as follows:

SIf the seating is too high on the valve face, use 30°

and 45° cutters to correct the seat.

1.0 – 1.4 mm

Z03988

2000 LEXUS LS400 (RM717U)

Author :

Date :

1229

EM–48

ENGINE MECHANICAL

–

CYLINDER HEAD

60°

45°

S

If the seating is too low on the valve face, use 60°

and 45° cutters to correct the seat.

1.0 – 1.4 mm

Z03989

(d)

(e)

Hand–lap the valve and valve seat with an abrasive com-

pound.

After hand–lapping, clean the valve and valve seat.

EM6331

Deviation

12.

(a)

INSPECT VALVE SPRINGS

Using a steel square, measure the deviation of the valve

spring.

Maximum deviation: 2.0 mm (0.079 in.)

If the deviation is greater than maximum, replace the valve

spring.

EM0988

Using vernier calipers, measure the free length of the

valve spring.

Free length: 54.05 – 54.15 mm (2.1279 – 2.1319 in.)

If the free length is not as specified, replace the valve spring.

(b)

EM0801

(c)

Using a spring tester, measure the tension of the valve

spring at the specified installed length.

Installed tension:

204 – 226 N (20.8 – 23.0 kgf, 45.9 – 50.7 lbf)

at 35.0 mm (1.378 in.)

If the installed tension is not as specified, replace the valve

spring.

EM0281

2000 LEXUS LS400 (RM717U)

Author :

Date :

1230

EM–49

ENGINE MECHANICAL

–

CYLINDER HEAD

13.

(a)

(b)

INSPECT CAMSHAFT FOR RUNOUT

Place the camshaft on V–blocks.

Using a dial indicator, measure the circle runout at the

center journal.

Maximum circle runout: 0.08 mm (0.0031 in.)

If the circle runout is greater than maximum, replace the cam-

shaft.

A03202

14. INSPECT CAM LOBES

Using a micrometer, measure the cam lobe height.

Standard cam lobe height:

Intake

Exhaust

42.610 – 42.710 mm (1.6776 – 1.6815 in.)

42.630 – 42.730 mm (1.6783 – 1.6823 in.)

Minimum cam lobe height:

Intake

EM2011

42.46 mm (1.6717 in.)

42.48 mm (1.6724 in.)

Exhaust

If the cam lobe height is less than minimum, replace the cam-

shaft.

15. INSPECT CAMSHAFT JOURNALS

Using a micrometer, measure the journal diameter.

Journal diameter:

Intake camshaft (A)

Others

30.984 – 31.000 mm (1.2198 – 1.2205 in.)

26.954 – 26.970 mm (1.0612 – 1.0618 in.)

If the journal diameter is not as specified, check the oil clear-

ance.

A

A

EM2538

A02880

A03222

2000 LEXUS LS400 (RM717U)

Author :

Date :

1231

EM–50

ENGINE MECHANICAL

–

CYLINDER HEAD

Red Painted

Mark

Knock Pin

Green

Painted

Mark

16. INSPECT CAMSHAFT TIMING TUBE JOURNALS

HINT:

There are 2 size of the camshaft timing tube journal diameter,

green and red painted mark accordingly. The mark is painted

on the face of the camshaft timing tube.

A03217

Using a micrometer, measure the journal diameter.

Journal diameter:

Green painted mark

Red painted mark

39.958 – 39.964 mm (1.5731 – 1.5734 in.)

39.964 – 39.970 mm (1.5734 – 1.5736 in.)

If the journal diameter is not as specified, check the oil clear-

ance.

A03203

EM3322

17. INSPECT CAMSHAFT GEAR SPRING

Using vernier calipers, measure the free distance between the

spring ends.

Free distance: 18.2 – 18.8 mm (0.712 – 0.740 in.)

If the free distance is not as specified, replace the gear spring.

18. INSPECT CAMSHAFT BEARINGS

Check that bearings for flaking and scoring.

If the bearings are damaged, replace the bearing caps and cyl-

inder head as a set.

19.

INSPECT CAMSHAFT TIMING TUBE AND CAMSHAFT

JOURNAL OIL CLEARANCE

LH Cylinder Head

RH Cylinder Head

Front

Mark

A or B

A03216

HINT:

There are 2 size of the camshaft timing tube journal oil clear-

ance, Marked ”A” and ”B” accordingly. The mark is stamped on

the top of the cylinder heads.

(a) Install the camshaft timing tube. (See page EM–58)

(b) Clean the bearing caps and camshaft journals.

(c) Place the camshafts on the cylinder head.

(d)

Lay a strip of Plastigage across each of the camshaft jour-

nals.

Plastigage

A03204

2000 LEXUS LS400 (RM717U)

Author :

Date :

1232

EM–51

ENGINE MECHANICAL

–

CYLINDER HEAD

(e)

Install the bearing caps. (See page EM–58)

Torque: 16 N·m (160 kgf·cm, 12 ft·lbf)

NOTICE:

Do not turn the camshaft.

(f)Remove the bearing caps.

A03205

(g)

Measure the Plastigage at its widest point.

Standard Oil clearance:

0.036 – 0.050 mm (0.0014 – 0.0020 in.)

Camshaft timing tube

Mark ”A”

Camshaft timing tube

Mark ”B”

Others

0.038 – 0.052 mm (0.0015 – 0.0021 in.)

0.030 – 0.067 mm (0.0012 – 0.0026 in.)

Maximum oil clearance:

A03206

Camshaft timing tube

Others

0.085 mm (0.0033 in.)

0.10 mm (0.0039 in.)

If the oil clearance is greater than maximum, replace the cam-

shaft. If necessary, replace the bearing caps and cylinder head

as a set.

(h) Completely remove the plastigage.

(i)Remove the camshafts.

(j)Remove the camshaft timing tube.

20. IF NECESSARY REPLACE CAMSHAFT TIMING TUBE

Select a camshaft timing tube according to mark on the cylinder

head.

Cylinder Head

Mark ”A”

Mark ”B”

Camshaft timing tube

Green painted mark

Red painted mark

21.

(a)

(b)

INSPECT CAMSHAFT THRUST CLEARANCE

Install the camshaft timing tube. (See page EM–58)

Install the camshaft. (See page EM–58)

(c)

Using a dial indicator, measure the thrust clearance while

moving the camshaft back and forth.

Standard thrust clearance:

Intake

Exhaust

0.060 – 0.100 mm (0.0024 – 0.0039 in.)

0.040 – 0.090 mm (0.0016 – 0.0035 in.)

Maximum thrust clearance:

Intake

A03207

0.13 mm (0.0051 in.)

0.12 mm (0.0047 in.)

Exhaust

2000 LEXUS LS400 (RM717U)

Author :

Date :

1233

EM–52

ENGINE MECHANICAL

–

CYLINDER HEAD

If the thrust clearance is greater than maximum, replace the

camshaft. If necessary, replace the bearing caps and cylinder

head as a set.

(d) Remove the camshafts.

(e) Remove the camshaft timing tube.

22. INSPECT CAMSHAFT GEAR BACKLASH

(a) Install the camshaft timing tube. (See page EM–58)

(b) Install the camshafts without installing the exhaust cam

sub–gear and front bearing cap. (See page EM–58)

(c)

Using a dial indicator, measure the backlash.

Backlash:

Standard

Maximum

0.020 – 0.200 mm (0.0008 – 0.0079 in.)

0.30 mm (0.0188 in.)

If the backlash is greater then maximum, replace the cam-

shafts.

(d) Remove the camshafts.

A03208

23.

(a)

INSPECT VALVE LIFTERS AND LIFTER BORES

Using a caliper gauge, measure the lifter bore diameter

of the cylinder head.

Lifter bore diameter:

31.000 – 31.016 mm (1.2205 – 1.2211 in.)

A03209

(b)

12.3 – 12.7 mm

(c)

Using a micrometer, measure the lifter diameter at the

valve lifter center line, 12.3 – 12.7 mm (0.484 – 0.500 in.)

from the valve lifter head.

Lifter diameter:

30.966 – 30.976 mm (1.2191 – 1.2195 in.)

Subtract the lifter diameter measurement from the lifter

bore diameter measurement.

Standard oil clearance:

Standard

Maximum

0.024 – 0.050 mm (0.0009 – 0.0020 in.)

0.07 mm (0.0028 in.)

A03201

If the oil clearance is greater than maximum, replace the lifter.

If necessary, replace the cylinder head.

2000 LEXUS LS400 (RM717U)

Author :

Date :

1234

EM–53

ENGINE MECHANICAL

–

CYLINDER HEAD

24.

(a)

INSPECT INTAKE MANIFOLD

Upper intake manifold:

Using a precision straight edge and feeler gauge, mea-

sure the surface contacting the lower intake manifold for

warpage.

Maximum warpage: 0.15 mm (0.0059 in.)

If warpage is greater than maximum, replace the upper intake

manifold.

A02881

Upper Intake Manifold Side

(b)

Lower intake manifold:

Using a precision straight edge and feeler gauge, mea-

sure the surface contacting the cylinder head and upper

intake manifold for warpage.

Maximum warpage: 0.15 mm (0.0059 in.)

If warpage is greater than maximum, replace the lower intake

manifold.

Cylinder Head Side

A02882

25. INSPECT EXHAUST MANIFOLD

Using a precision straight edge and feeler gauge, measure the

surface contacting the cylinder head for warpage.

Maximum warpage: 0.50 mm (0.0197 in.)

If warpage is greater than maximum, replace the manifold.

P21864

Measuring Point

26. INSPECT CYLINDER HEAD BOLTS

Using vernier calipers, measure the thread outside diameter of

the bolt.

Outside diameter:

Standard

Minimum

9.770 – 9.960 mm (0.3846 – 0.3921 in.)

9.60 mm (0.3780 in.)

If the diameter is less than minimum, replace the bolt.

2.5 mm

(0.98 in.)

2000 LEXUS LS400 (RM717U)

A02825

Author :

Date :

1235