high brake light

https://www.clublexus.com/forums/ls-1st-and-2nd-gen-1990-2000/511347-third-brake-light-ls400-3rd-gen-light-bulb-fix.html

No one seemed to have this fix for the third brake light (high mounted stop light) on the third generation LS400 (1998-2000), so here it is. 
Here is what is looks like when you lift up and pull out. It does take quite a bit of force to pull this out. Make sure you lift highenough to have clearance on the red plastic. I used two screwdrivers on each side to lift the light up and then grab on the sides and pull out level.
Next, turn counter clockwise to remove light fixture. Now pull bulb out--it does not unscrew.
Best to replace both bulbs once you're in there.
Check your connections and test the lights to make sure they are working. 
Replace the whole unit by pushing back in place. This goes easy and fits snug.
My bulb lasted over 11 years. Pretty good. The Lexus dealer wanted $10 per bulb. Local auto parts store did it for $3.80
Hope this helps you out.
Bob