2. INSPECT FRONT NO. 3 SPEAKER ASSEMBLY

(a) Measure the resistance according to the value(s) in the table below.

Standard resistance:

| Tester Connection | Condition | Specified Condition |
| --- | --- | --- |
| 1 - 2 | Always | Approx. 6 Ω *1 |
| 1 - 2 | Always | 5 to 7 Ω *2 |

HINT:

- *1: for standard models.

- *2: for 19 speakers models.

- If the result is not as specified, replace the speaker assembly.

| 1. INSPECT FRONT NO. 2 SPEAKER ASSEMBLY (a) Measure the resistance according to the value(s) in the table below. Standard resistance: Tester Connection; Condition; Specified Condition. 1 - 2; Always; Approx. 6 Ω *1. 1 - 2; Always; 5 to 7 Ω *2. HINT: - *1: for Standard models. - *2: for 19 speakers models. If the result is not as specified, replace the speaker assembly. |  |
| --- | --- |

- REMOVAL

HINT:

- Use the same procedures for the LH side and RH side.

- The procedures listed below are for the LH side.

1. REMOVE COWL TOP VENTILATOR LOUVER RH

2. DISCONNECT CABLE FROM NEGATIVE BATTERY TERMINAL

CAUTION:

Wait at least 90 seconds after disconnecting the cable from the negative (-) battery terminal to prevent airbag and seat belt pretensioner activation.

NOTICE:

After the engine switch is turned off, the HDD navigation system requires approximately 6 minutes to record various types of memory and settings. As a result, after turning the engine switch off, wait 6 minutes or more before disconnecting the cable from the negative (-) battery terminal.

| 3. REMOVE NO. 1 SPEAKER HOLE COVER (a) Using a screwdriver, detach the 4 clips and remove the hole cover. HINT: Tape the screwdriver tip before use. |  |
| --- | --- |

4. REMOVE FRONT NO. 3 SPEAKER ASSEMBLY

(a) Remove the 2 bolts.

(b) Remove the speaker and disconnect the speaker connector.

NOTICE:

Do not touch the cone part of the speaker.