48 Months or 40,000 Miles

❏ Replace engine oil with 0W-20 oil; replace oil filter1

❑ Replace spark plugs 2

2 Required under the terms of the Emission Control Warranty

❑ Replace cabin air filter

❏ Reset oil maintenance reminder, if oil was changed

❑ Reset scheduled maintenance reminder

❏ Rotate tires

❏ Visually inspect brake pads, calipers and rotors

❑ Inspect wiper blades

❑ Check installation of driver’s floor mat

❏ Road test vehicle If you drive primarily under any of these special operating conditions, perform these additional items: Driving on dirt roads or on dusty roads:

❏ Inspect axle shaft boots

❏ Inspect ball joints and dust covers

❏ Inspect engine air filter

❏ Inspect steering linkage and boots

❏ Re-torque nuts and bolts of drive shaft (AWD)

❏ Tighten nuts and bolts on chassis Towing a trailer3 , using a car-top carrier or heavy vehicle loading: ❏ Re-torque nuts and bolts of drive shaft (AWD)

❏ Replace transmission fluid

❏ Tighten nuts and bolts on chassis Driving on roads on which road salt has been applied:

❏ Re-torque nuts and bolts of drive shaft (AWD) 4