The I16 plug should look like this:

Looking at the pins.....

X indicates no pin

X      4       3       X        X

X      8                X        6

13     12              X       10

3 goes to I18-9   	ALSO M8-8           	  		WORKS

4 goes to I18-8   ALSO R28-2 AND M8-8	            	WORKS

6 goes to I-18-4                                           		WORKS

8 goes to I-18-14                                                          WORKS

10 goes to I-18-11                                       		WORKS

12 goes to I-18-12                                      		WORKS

13 goes to I-18-5                                        		WORKS

------------------------------------------------------------------------------------------------

///AUDIO PLUGS\\\

N3 is the 18 pin plug on the Navigation computer

S14 is the "Y" harness AT the amplifier.  (or "T" cable)

N3-1 goes to S14-10   FEMALE

N3-10   goes to S14-9   FEMALE

|

N3-2   goes to S14-10    MALE

N3-11   goes to S14-9    MALE

|

Remember on the S14 plug, the female plug will look like this

1   2             3     4

5   6   7   8   9    10

Male will be the opposite

4     3             2    1

10   9   8   7   6    5